- Installation

Run Setup.exe

Click Next

Enter your information, Click Next

Click Next to install or click Change to install to a different folder

Click Install to begin installation

Installing…

Install completed. Check Launch the driver installation and Click Finish

Click Install to Install the Driver for CashDrawer.

Driver installation is complete. Click OK and Click Exit to complete the installation

- Driver Install

You can install the driver when installing the program. If it is not installed at that time, you can reinstall it as follows.

Menu Start -> Opos Adk -> DriverInstall

Click Install

- Setup

Menu Start -> Opos Adk -> SetupOPOS

  - Add New Device

Expand list -> Focus on CashDrawer -> Click Menu Add or toolsbar Add

Select Device Name -> Enter a Logical Device Name as needed -> Click OK to finish

  - Edit Device Settings

Expand list -> Focus on Device Name (As SKYLAKE_CR) -> Click Menu Edit or toolsbar Edit

Enter a new Logical Device Name -> Click OK to finish.

(Modify Logical Device Name only)

  - Delete Device

Expand list -> Focus on Device Name (As SKYLAKE_CR) -> Click Menu Delete or toolsbar Delete

Click Yes to delete device.

  - CheckHealth

Expand list -> Focus on Device Name (As 6100) -> Click Menu CheckHealth or toolsbar CheckHealth

If there is an error, the following prompt will pop up:

(ResultCodeExtended refer Part 5)

If there are no errors, the CheckHealth window will pop up:

CheckHealth is completed and closed, the following prompt pops up:

- Test

Menu Start -> Opos Adk -> CashDrawerTest

- Select the device name to open.

- [Open] Opens a device for subsequent I/O.

- [Claim] Requests exclusive access to the device.

[Release] Releases exclusive access to the device.

- [Device Enable] If true, the device is in an operational state. If false, the device has been disabled.

- [Close] Releases the device and its resources.

If the DeviceEnabled property is true, then the device is disabled.

If the Claimed property is true, then exclusive access to the device is released.

- [Read Status] If true, the drawer is open. If false, the drawer is closed.

- [Open Drawer] Opens the drawer.

- [Wait Drawer Close] Waits until the cash drawer is closed. If the drawer is still open after beepTimeout milliseconds, then the system alert beeper is started.

- [Status Update] When the status of the Cash Drawer changes to inform the application of the current state.

- [Freeze Event] If true, the UnifiedPOS Control will not deliver events (Status of the Cash Drawer changes). If false, the application allows events to be delivered.

- ResultCodeExtended

| 10001 | Error loading dynamic library |
| --- | --- |
| 10002 | Error loading driver |
| 10003 | Wrong model |
| 10999 | Unknown error |