Elanda T5 Cash Drawer

Installation & Setup Guide

For use with the CashDrawer opener application on Windows

# Overview

This guide walks through the complete setup of the Elanda T5 cash drawer on a new Windows computer. The process installs two software components and configures a logical device name that matches the opener application filename.

What you will need:

- OPOS_CCOs_1.14.001.msi — Monroe OPOS Common Control Objects

- Elanda OPOS ADK — Setup.exe and driver package

- CashDrawer opener application (e.g. cashdrawer1.exe)

📌 All three items should be included in the ZIP file provided to you. Extract the ZIP to your Desktop before starting.

# Step 1 — Install OPOS Common Control Objects

The OPOS CCOs are the standard Windows COM layer that all OPOS hardware communicates through. This must be installed before the Elanda driver.

- Locate OPOS_CCOs_1.14.001.msi in the extracted ZIP folder.

- Right-click the file and select Run as Administrator.

- Click Next through the installer, accepting all defaults.

- Click Install, then Finish when complete.

📌 If Windows asks for permission via a UAC prompt, click Yes to allow the installation.

# Step 2 — Install Elanda OPOS ADK

The Elanda OPOS ADK installs the hardware service object and driver that allows Windows to communicate with the T5's built-in cash drawer port.

- Locate Setup.exe in the Elanda OPOS ADK folder.

- Right-click and select Run as Administrator.

- Click Next, enter your information if prompted, click Next again.

- Click Next to accept the default install folder, then click Install.

- When installation completes, check the box labeled Launch the driver installation and click Finish.

- A Driver Installation window will appear — click Install.

- When the driver finishes, click OK, then click Exit.

📌 Both the ADK install and the driver install must complete successfully before continuing.

# Step 3 — Configure the Cash Drawer Device

This step registers a logical device name in OPOS that links your opener application to the physical cash drawer port on the T5.

- Go to Start → OPOS ADK → SetupOPOS.

- In the left panel, expand the list and click on CashDrawer.

- Click Add from the menu or toolbar.

- In the Device Name dropdown, select ELKHART LAKE_CR.

- In the Logical Device Name field, type the name of your opener exe without the .exe extension.

Examples:

- If your exe is named cashdrawer1.exe → type cashdrawer1

- If your exe is named cashdrawer2.exe → type cashdrawer2

- Click OK to save.

📌 The Logical Device Name must exactly match the exe filename (without .exe) — spelling, capitalisation, and all.

# Step 4 — Test the Configuration

Before using the opener application, verify the hardware is working correctly using the built-in test tool.

- Go to Start → OPOS ADK → CashDrawerTest.

- In the dropdown at the top, select the logical device name you just created (e.g. cashdrawer1).

- Click each button in order and confirm the result codes shown below.

| Button | Expected Result |
| --- | --- |
| Open | Open 0 — device connected successfully |
| Claim | ClaimDevice 0 — exclusive access granted |
| Device Enable | DeviceEnabled True — device is ready |
| Open Drawer | OpenDrawer 0 — drawer opens physically |
| Device Enable | DeviceEnabled False — disable before release |
| Release | ReleaseDevice 0 — access released |
| Close | Close 0 — device closed cleanly |

📌 If any step returns a non-zero code, do not proceed. Refer to the Troubleshooting section at the end of this guide.

# Step 5 — Run the Opener Application

Once the test passes, the opener application is ready to use.

- Copy cashdrawer1.exe (or your named exe) to a convenient location on the computer, such as C:\POS\.

- Double-click the exe to run it.

- The console window will display the device name it is targeting and confirm success.

Expected output:

Opening drawer for device: cashdrawer1

[OK] Drawer opened!

📌 The exe can be called from any other application or script simply by running it as a process. No parameters are needed — the filename is the configuration.

# Setting Up Additional Cash Drawers

To add a second cash drawer on the same machine, repeat Step 3 with a different logical device name and a different hardware device, then name the exe to match.

- In SetupOPOS, add a new CashDrawer device — select ELKHART LAKE_B_CR, name it cashdrawer2.

- Copy the exe and rename the copy to cashdrawer2.exe.

- Running cashdrawer2.exe will open the second drawer.

📌 Each physical drawer port on the T5 corresponds to a different device name in the ELKHART LAKE series (A, B, etc.).

# Troubleshooting

Error: Class not registered (0x80040154)

The OPOS CCOs are not installed, or the exe is running as 64-bit. Ensure OPOS_CCOs_1.14.001.msi was installed as Administrator. The opener exe must be compiled or published targeting x86 (32-bit).

ClaimDevice returns 106 / ResultCodeExtended 10002

The Elanda driver is not installed correctly. Go to Start → OPOS ADK → DriverInstall, click Install, and try again.

ClaimDevice returns 101

Another application already has the device claimed. Close any other POS software or the CashDrawerTest app and try again.

OpenDrawer returns 105

Device Enable was not clicked before Open Drawer. Always enable the device before attempting to open the drawer.

Drawer does not open physically but returns code 0

Check that the cash drawer RJ11 cable is firmly connected to the port on the back of the Elanda T5. Try a different cable if available.